TOWNSHIP OF WANTAGEPRIVATE 

An ordinance to amend the driveway construction ordinance of the revised general ordinances of the township of wantage


Ordinance No. 2003-18 

WHEREAS, the Wantage Township Engineer and the Wantage Township Construction Official have recommended that the Township Driveway Construction Ordinance be amended to insure that the responsibility and expense for verifying consistency with the Township Driveway requirements be at all times on the individual property owner wishing to construct a new driveway;


WHEREAS, the Wantage Township Committee believes it is in the best interest of the Township to adopt this revision to the Driveway Construction Ordinance.


NOW, THEREFORE, BE IT ORDAINED by the Mayor and Township Committee of the Township of Wantage, that Chapter XIX, Section 19-2, of the General Ordinances of the Township of Wantage, shall be and is amended as follows:


Section 1.
Section 19-2a. shall be amended to read as follows:

“All applications for a driveway access permit shall be accompanied by a certified survey prepared and sealed by a New Jersey licensed engineer or surveyor containing the following details:  

1. driveway location and dimensions;

2. all existing features shall be shown at a scale of 1 inch = 50 feet or larger;

3. driveway slope per 100 feet sustained;

4. centerline profile if grade exceeds eight (8) percent at any point along entire driveway; and

5. sight distance for all driveway egress points designed in accordance with Section 19-2b.5.”


Section seq level0 \h \r0 

seq level1 \h \r0 

seq level2 \h \r0 

seq level3 \h \r0 

seq level4 \h \r0 

seq level5 \h \r0 

seq level6 \h \r0 

seq level7 \h \r0 2.
Section 19-2b.2.b. shall amended to read as follows:

“Surface material – Hot Mix Asphalt Mix I-5, two inches thick, compacted.”


Section 3.  
Section 19-2b.5. shall be amended to read as follows:

“The sight distance shall be taken 3½ feet above a point in the centerline of the proposed driveway, 10 feet from the edge of the road, to a point 3½ feet above the centerline of the approaching lane.  There shall be a one foot sight clearance between the sight line and finished grade.  All necessary grading and removal of vegetation within the sight triangle shall be indicated on plans to conform with the following criteria:

	Design Speed of Road
	Required Sight Distance

	20 mph
	120 feet

	25 mph
	150 feet

	30 mph
	200 feet

	35 mph
	250 feet

	40 mph
	325 feet

	50 mph
	475 feet



Section 4. 
Section 19-2e. shall be amended to read as follows:

“All driveways shall be designed in accordance with the preceding criteria.  Any deviation must be approved by the Township Engineer or the Land Use Board as the case may be.”


Section 5.
Section 19-2f. shall be amended to read as follows:

“Unless otherwise required by the Township Engineer or the Land Use Board, all driveways that slope away from the road, or are determined to have no drainage effect on the road, shall be paved with shale, stone or like material for a minimum depth of four (4) inches”.


Section 6.
Section 19-2h. shall be amended to read as follows:

“Driveway access permit fee shall be $50.00.  In addition, if required by the Township Construction Official or the Land Use Board, the applicant shall put a cash deposit, performance bond and/or other contractual guarantee for the restoration of the Township streets, shoulders and slopes.  In addition, if required by the Township Construction Official or Land Use Board, a cash deposit for inspection fees and professional review fees, if deemed necessary in the particular case, shall be posted in an amount up to $500.00.


Section 7.
A new section 19-2k. shall be added and shall read as follows:

“Following driveway construction, a New Jersey Licensed Engineer or Land Surveyor shall submit an ‘as-built’ driveway plan certifying and showing the following information:

A. Driveway location from street to dwelling;

B. Driveway profile from street to dwelling;

C. Sight distance profiles designed in accordance with Section 19-2b.5;

D. Cross drains.”


Section 8.
A new section 19-2l. shall be added and shall read as follows:

“No more than two single family residential lots may be accessed via a single common driveway egress point.  A common driveway shall be constructed to the following standards:

A. The width shall be 20 feet;

B. The maximum grade shall be in accordance with §19-2b.1

C. That portion of the driveway common to both lots shall be paved for its entire length.

D. A common driveway shall be constructed with Dense Graded Aggregate base course, 4” thick, and Hot Mix Asphalt, 2” thick, Mix I-5.”


Section 9.  Should any section, paragraph, sentence, clause or phrase of this Ordinance be declared unconstitutional or invalid for any reason by any Court of competent jurisdiction, such provision(s) shall be deemed severable and the remaining portions of this Ordinance shall remain in full force and effect.


Section 10.
All ordinances or parts of ordinances or resolutions that are inconsistent with the provisions of this Ordinance are repealed to the extent of such inconsistency.


Section 11.
This ordinance shall take effect after publication and passage according to law.




















__________________________





                       Jeffrey Parrott, Mayor

ATTEST:

_______________________________________

James R. Doherty, Administrator/Municipal Clerk 


NOTICE

TAKE NOTICE that the above entitled ordinance was introduced at a regular meeting of the Mayor and Township Committee of the Township of Wantage, County of Sussex, New Jersey, held on September 25, 2003, and will be considered for final passage after public hearing at a regular meeting of the Mayor and Township Committee to be held on October 9, 2003 at 8:00 p.m. in the Municipal Building, Route 23, Wantage, New Jersey.  During the preceding week, copies of the ordinance will be made available at the Clerk's office in the Municipal Building to members of the general public.


______________________________________







James R. Doherty, Administrator/Municipal Clerk

MDC
N:\USERS\Clients\3020-904\Driveway Ordinance Amend 8-12-03.doc
09/03/03

Revised 10/21/04












PAGE  
1

